CS4188/CS5188 Virtual Reality Technologies and Applications

期末复习完整文稿（可打印版）

编制依据：复习路径.md + 复习Prompt.md 两份核心指引文档 覆盖章节：Chapter 2 (Input Devices) + Chapter 3 (Graphical Displays) + Chapter 4 (Sound and Haptics) 适用场景：闭卷考试，无计算题，Short Questions + Two-level MCQ + Long Question 答题风格：Definition → Mechanism → Pros/Cons → Application → Trade-off

【考试总框架】三大主轴

| 主轴 | 核心问题 | 关联章节 |
| --- | --- | --- |
| Input-Output Devices | 用户如何感知和作用于VR世界？ | Ch2, Ch3, Ch4 |
| Architecture | 系统如何实时更新、渲染、同步、保持低延迟？ | Ch5 (Pipeline), Ch9 (DVE) |
| Applications | VR在现实任务中为何有用、如何设计？ | Ch8 |

考试结构： - Short Questions：约10道，约50分 - Two-level Multiple-Choice：约10道，约25分
- Long Question：1道，约25分

答题万能结构：

Definition（定义）→ Mechanism（机制）→ Pros/Cons（优缺点）
→ Application（应用场景）→ Trade-off/Limitation（权衡与局限）

第一部分：INPUT DEVICES（输入设备与追踪）

Chapter 2 - 对应课件：2 - Input Devices.pdf

1. 核心定义 / Core Definitions

1.1 Tracker（追踪器）

英文定义：A device to measure (or to track) the position and/or orientation of an object in real-time. 中文定义：实时测量物体位置和/或方向的设备。 考试要点： - Typical targets：head, hands, limbs（头、手、肢体） - Specified relative to：reference / world coordinate system（参考/世界坐标系） - VR中最常追踪：用户头部（确定视角）、手部（交互）、肢体（全身追踪）

1.2 DOF - Degrees of Freedom（自由度）

英文定义：The degrees of freedom indicate the number of independent motions (or measurements) to be tracked. 中文定义：自由度表示需要追踪的独立运动或测量数量。

| DOF | 示例设备 | 考试关键词 |
| --- | --- | --- |
| 2 DOF | 2D mouse | 平面移动 |
| 3 DOF | Accelerometer（测位置）/ Gyroscope（测方向） | 仅位置或仅方向的某些维度 |
| 6 DOF | VR headset / controller / 智能手机 | Position (x,y,z) + Orientation (roll, pitch, yaw) |

⚠️ 高频考点：6 DOF = 3 position + 3 orientation，是VR头显和手柄的标准配置。

1.3 坐标变换链 / Coordinate Transformation Chain

核心公式：

v2D = Mproj × Mview × Mworld × v3D

| 矩阵 | 英文全称 | 中文含义 | 作用 |
| --- | --- | --- | --- |
| Mworld | Model Matrix | 模型矩阵 | 3D point in model local → world coordinate（模型坐标系→世界坐标系） |
| Mview | View Matrix | 视图矩阵 | 3D point in world → view coordinate（世界坐标系→观察者视角） |
| Mproj | Projection Matrix | 投影矩阵 | 3D point in view → 2D image（视图坐标系→二维图像） |

考试答法： > To determine the user’s viewpoint in VR, we attach a tracker to the user’s head. The tracker’s position gives T(-eye) for translation, and its orientation gives R(-θroll, -θpitch, -θyaw) for rotation. Combined, these form the Mview matrix that transforms world coordinates to the user’s view.

2. Tracking Performance 四指标 / Four Metrics

| 指标 | 英文 | 定义 | 关键特征 | 常见陷阱 |
| --- | --- | --- | --- | --- |
| Accuracy | 准确度 | Measured value close to actual value | 随距离原点增加而下降；决定operating range | 不是constant，空间误差 |
| Jitter | 抖动 | Output changes even when object is stationary | 随机波动，围绕true value | “静止也乱跳” |
| Drift | 漂移 | Inaccuracy grows over time | 时间累积误差，需secondary tracker定期reset | “越久越偏” |
| Latency | 延迟 | Delay between action and signal output | 大延迟导致simulation sickness（头痛、恶心、眩晕） | “动作到反馈的延迟” |

⚠️ 极易混淆： - Jitter = 静止时的随机噪声（空间上的不稳定） - Drift = 时间上的累积偏移（越久误差越大） - Latency = 时间延迟（动作发生到系统响应的时间差） - Accuracy = 空间准确度（测量值与真实值的接近程度）

考试标准句： > A large delay between the motion and the visual feedback causes simulation sickness, such as headaches, nausea, and dizziness.

3. 六大Tracker类型对比 / Six Tracker Types Comparison

3.1 总表（必背）

| Type | Principle | Advantages | Drawbacks | Representative |
| --- | --- | --- | --- | --- |
| Mechanical | Links + sensorized joints | Accurate, small latency, no line-of-sight, unlimited workspace, small drift | Intrusive, heavy (5-7kg), limited motion range, mechanical oscillation, fatigue | Gypsy Motion Capture |
| Magnetic | Transmitter magnetic field → receiver | Lightweight, no line-of-sight restriction | Metal interference, range limitation, ambient error ∝ d⁴ | Polhemus Fastrak, Ascension MotionStar |
| Ultrasonic | Time-of-flight of sound | Cheap, simple, small/light, no metal interference | Low update rate (~50Hz), echoes, line-of-sight, air absorption, temp/humidity dependent | Tutorial 1 |
| Optical | Cameras + markers/beacons | High precision, standard mocap, high update rates | Occlusion, calibration, marker ambiguity, sensitivity degrades with distance | MotionAnalysis, HiBall |
| Inertial/IMU | Gyro + accelerometer + magnetometer | Small, cheap, high update rate, no line-of-sight, good short-term | Gyro drift, accelerometer noise, magnetic interference, needs fusion/reset | MPU-9250 (9 DoF) |
| 3D Navigation | 6 DOF input devices | Direct manipulation, high sampling rate | Sensor coupling, calibration, fatigue | Magellan, MicroScribe |

3.2 Mechanical Trackers（机械式追踪器）

英文要点： > A serial or parallel kinematic structure composed of links, interconnected using sensorized joints. The system knows the kinematic chain structure in advance.

中文要点： 由连杆和传感器关节组成的串/并联运动结构，系统预先知道运动链结构。

优点： - Accurate（精确） - Small latency（低延迟） - No line-of-sight restriction（无视线遮挡问题） - Long recording possible – small drift（可长时间记录，漂移小） - Workspace unlimited (motion relative to base)（工作空间无限，相对于基座）

缺点： - Limited range of motion（运动范围受机械结构限制） - Unwanted mechanical oscillation due to inertia（质量越大，惯性振荡越严重） - Heavy weight → fatigue & diminished immersion（重量大→疲劳、沉浸感下降） - Interferes with user’s motion, reduces freedom（干扰用户运动）

代表设备：Gypsy Motion Capture - 15 joints measured by 42 potentiometers - Accuracy: 0.08° - Weight: 5-7kg - Includes footstep micro-switches for walking/running capture - Gypsy 2.5 adds hip gyroscope for rotation tracking

考试标准句： > Mechanical trackers are intrusive to motion, while non-contact motion trackers (magnetic, ultrasonic, optical, inertial) do not have this problem.

3.3 Magnetic Trackers（磁式追踪器）

原理： > Use magnetic field produced by a stationary transmitter to determine real-time position of a moving receiver.

Transmitter结构：Three mutually orthogonal coils wounded on ferromagnetic cube（三个相互正交线圈绕在铁磁立方体上）

Receiver结构：Three small orthogonal coils (AC) or magnetometers (DC)

AC vs DC Magnetic Trackers（高频考点）

| Feature | AC Magnetic | DC Magnetic |
| --- | --- | --- |
| Field type | Alternating field 7-14kHz | Pulsed DC field |
| Eddy current problem | Severe – changing field induces eddy currents in metals → distorted field | Alleviated – short delay between pulses allows eddy currents to disappear |
| Metal interference | All metals (especially copper, ferromagnetic) | Unaffected by non-ferromagnetic metals (brass, aluminium, stainless steel); still affected by ferromagnetic metals and copper |
| Representative | Polhemus Isotrak (~30ms latency, 1° jitter) | Ascension Flock of Birds (144Hz, parallel processing) |

关键公式直觉： - Magnetic field intensity ∝ 1/d³（磁场强度与距离立方成反比） - Position error ∝ d⁴（位置误差与距离四次方成正比）

考试标准句： > When the strength of the magnetic field changes, eddy currents are induced in surrounding metal. These currents generate new magnetic fields that oppose the change of the original magnetic field, degrading tracker accuracy.

3.4 Ultrasonic Trackers（超声波追踪器）

原理： > Uses ultrasonic signal produced by stationary transmitter to determine real-time position of moving receiver. Time-of-flight × speed of sound = distance.

结构： - Transmitter: three ultrasonic speakers mounted ~30cm apart on rigid triangle frame - Receiver: three ultrasonic microphones on smaller rigid triangle frame

测量过程： - Each speaker activated in cycle - Three distances from one speaker to three microphones calculated - Total nine distances → determine position and orientation

性能： - Update rate: ~50 data sets/sec（不到magnetic tracker的一半） - Operating range: typically 1.52m from transmitter - 低更新率原因：需等待5-100ms让echoes消散

优点： - No interference with metal（不受金属干扰） - Small and lightweight（小巧轻便） - Relatively inexpensive（相对便宜）

缺点： - Line-of-sight restriction（需要视线） - Accuracy depends on sound velocity（准确度依赖声速，而声速受温度/压力/湿度影响） - Susceptible to noise（易受噪声影响，如墙壁反射） - Low update rate（更新率低）

Tutorial 1 关联： > Ultrasonic tracking estimates distances by measuring sound time-of-flight, then reconstructs position and orientation geometrically. Two speakers can locate one microphone; two microphones can determine tracker orientation.

3.5 Optical Trackers（光学追踪器）

定义： > A non-contact position measurement device based on optical sensing, calculating real-time position/orientation of an object. Requires direct line of sight.

特点： - High update rates（高更新率） - Small latency（低延迟）

Outside-Looking-In vs Inside-Looking-Out

|  | Outside-Looking-In | Inside-Looking-Out |
| --- | --- | --- |
| Sensor position | Fixed cameras outside | Camera on user/object |
| Markers | On user/object | On walls/environment |
| Advantage | Standard mocap, high precision | Theoretically infinite workspace, sensitive to orientation changes |
| Disadvantage | Occlusion, calibration needed | Requires environment setup |
| Representative | MotionAnalysis (8 IR cameras) | HiBall 3000 (2000 data sets/sec) |

Motion Capture Pipeline（5步）： 1. Calibration – capture special markers with known positions 2. Motion Capture – actor performs action 3. 3D Position Reconstruction – at least 2 cameras see each marker 4. Fitting the Skeleton – associate markers to predefined skeleton 5. Post Processing – noise removal, speed change, motion editing

优点： - High precision: standard method for motion capturing - Many techniques improve precision: camera calibration, tracking algorithms

缺点： - Occlusion problem: body parts may obscure each other - If 3D model available, can significantly reduce error

Tracking sensitivity degrades when： - Distances among markers decrease - Distance between user and camera increases - Distance between adjacent cameras increases

考试标准句： > With the captured 3D motion data, a virtual object can be programmed to follow the movement of a real object. By exaggerating the captured information, it is even possible to produce more interesting effects.

3.6 Inertial Trackers / IMU（惯性追踪器）

三种传感器：

| Sensor | Measures | Advantage | Disadvantage | Complementary Role |
| --- | --- | --- | --- | --- |
| Gyroscope | Angular velocity (°/s) | Short-term accurate | Drift over time (error ∝ t) | Short-term orientation |
| Accelerometer | Linear acceleration (m/s²) | Long-term stable, no drift | Short-term noisy | Long-term position (double integration) |
| Magnetometer | Magnetic field (μT) | Absolute direction reference | Affected by metal, latitude/longitude dependent | Direction correction |

IMU Example：InvenSense MPU-9250 (9 DoF) - 3-axis gyroscope + 3-axis accelerometer + 3-axis magnetometer - 9 × 16-bit ADCs for digitizing

核心互补关系： > Gyroscope is accurate short-term but drifts long-term. Accelerometer is reliable long-term but noisy short-term. Their measurements are complementary.

误差累积： - Gyroscope bias → orientation error increases proportionally with time - Accelerometer bias → position error increases with square of time - Commercial devices: 40mm error in 2 seconds - Expensive devices: 40mm error in 200 seconds

解决方案： > Periodically reset the error using another type of tracker (e.g., optical or magnetic).

优点： - Unlimited range of tracking（追踪范围无限） - No line-of-sight constraints（无视线限制） - Low jitter（低抖动）

缺点： - Rapidly accumulating errors or drift（快速累积误差） - Gyroscope bias → orientation error ∝ time - Accelerometer bias → error ∝ time²

3.7 3D Navigation/Manipulation Devices

| Device | Principle | Features | Disadvantages |
| --- | --- | --- | --- |
| Logitech Magellan | Sensorized cylinder measures 3 forces + 3 torques via photo sensors detecting LED light distortion | 6 DOF input | Sensor coupling: translation may cause unwanted rotation |
| MicroScribe 3D | 6 rotary joints, 6 DOF probe, direct kinematics calculation | Counterbalance near base reduces fatigue; >1000 samples/sec; binary foot pedal switch | Kinematic arm accumulates errors from base to tip; calibration needed |

4. Gesture Devices（手势设备）

| Device | Principle | Measurement | Resolution |
| --- | --- | --- | --- |
| Pinch Gloves | Conductive fibre patches at fingertips/back of fingers/palm | Binary contact (on/off) | Binary |
| 5DT Data Glove | Fibre-optic sensor loop, light intensity change on bending | Finger joint angles | 256 positions (8-bit A/D) |
| CyberGlove | Thin electrical strain gauges on elastic nylon glove | 18-22 sensors: finger flexion, abduction, wrist yaw/pitch | Differential voltage amplification |
| Didjigloves | Capacitive bend sensors (conductive polymer layers) | 10 sensors, overlapping surface ∝ bending angle | Capacitance measurement |

CyberGlove popularity原因： - Large number of sensors - Good programming support - Extensions into haptic gloves

Normalization formula（5DT Data Glove）：

out_val = (raw - raw_min) / (raw_max - raw_min) × 256

5. Tutorial 关联原理

Tutorial 1: Ultrasonic Tracker

- 原理：Sound time-of-flight × speed of sound = distance

- 定位：Two speakers → one microphone position (triangulation)

- 方向：Two microphones → tracker orientation

- 考试答法：Ultrasonic tracking estimates distances by measuring sound time-of-flight, then reconstructs position and orientation geometrically.

Tutorial 2: DC Magnetic Tracking

- 原理：B = μ₀I/(2πr), magnetic field strength与距离成反比

- 定位：Multiple wires with known magnetic readings → solve for position

- 方向：Right-hand rule判断磁场方向

- 考试答法：Magnetic field strength decreases with distance from the wire. By measuring magnetic field at different locations, the position relative to the wire can be determined.

Tutorial 3: Locating Markers with Cameras

- 原理：3D point projects to 2D image plane via camera projection

- 定位：Multiple cameras observing same marker → triangulate 3D position

- 退化情况：Points and camera center collinear → no unique solution

- 考试答法：Camera calibration estimates camera parameters. Multiple cameras observing the same marker allow 3D reconstruction through triangulation.

6. M2 一页纸总结 / One-Page Summary

TRACKER = real-time position/orientation measurement
↓
PERFORMANCE = accuracy (准不准) / jitter (静止抖) / drift (越久越偏) / latency (延迟致晕)
↓
TYPES:
  Mechanical = accurate, low latency, but intrusive & heavy
  Magnetic = no line-of-sight, but metal-sensitive (AC: eddy current; DC: ferromagnetic/copper)
  Ultrasonic = cheap/simple, but low update rate, line-of-sight, air absorption
  Optical = precise, but occlusion/calibration (outside-in vs inside-out)
  Inertial = small/fast/cheap, but drift/noise → needs sensor fusion + periodic reset
↓
IMU: gyro (short-term accurate, long-term drift) + accelerometer (long-term stable, short-term noisy) + magnetometer (direction, metal-sensitive)
↓
GESTURE: Pinch (binary) → 5DT (fibre-optic, 256 positions) → CyberGlove (strain gauges, 18-22 sensors)

第二部分：OUTPUT DEVICES - GRAPHICAL DISPLAYS（图形显示输出）

Chapter 3 - 对应课件：3 - Output Devices (Graphical Displays).pdf

1. Human Visual System（人眼视觉系统）

1.1 视网膜结构 / Retina Structure

| 结构 | 英文 | 功能 | VR关联 |
| --- | --- | --- | --- |
| 视网膜 | Retina | 1.26亿 photoreceptors，uneven distribution | 决定显示分辨率需求分布 |
| 黄斑/中央凹 | Fovea | Central area, high-resolution, color perception | VR需高分辨率中心区域 |
| 周边视觉 | Peripheral vision | Low-resolution, motion perception | 大FOV增强presence，peripheral对motion敏感 |
| 视杆细胞 | Rods | Night vision (暗视觉) | — |
| 视锥细胞 | Cones (L/M/S) | Color vision: L-red, M-green, S-blue | 三色感知，M-cone最敏感 |

光敏曲线： - Luminous-efficiency function V(λ) = R(λ) + G(λ) + B(λ) - 人眼对绿光最敏感（M-cone在中间光谱最活跃）

1.2 Field of View (FOV) / 视场角

| 类型 | 数值 | 考试意义 |
| --- | --- | --- |
| 单眼 | ~150° horizontal, 135° vertical | 单眼覆盖范围 |
| 双眼合并 | ~180° horizontal, 135° vertical | 总覆盖范围 |
| 双眼重叠区 | ~120° horizontal | Stereo depth perception区域 |

考试标准句： > Larger FOV increases immersion and presence, but requires higher rendering resolution and larger display coverage, increasing technical cost.

1.3 Depth Perception / 深度感知

Near Object Cues（近物深度线索）

| Cue | 英文机制 | 中文解释 | 关键区分 |
| --- | --- | --- | --- |
| Convergence | Rotate both eyes to focus on nearby point | 双眼会聚：两只眼睛转向同一近物 | 肌肉动作，双眼夹角变化 |
| Accommodation | Adjustment of ciliary muscles to focus | 调节：睫状肌调整晶状体形状对焦 | 晶状体变形，focus distance变化 |
| Disparity | Horizontal shift of objects between two eyes | 视差：同一物体在左右眼图像中的水平位置差 | 大脑用此估计深度 |
| Parallax | Nearby objects appear to move more than distant ones when viewer moves | 运动视差：观察者移动时，近物比远物移动更多 | 相对运动差异 |

Far Object Cues（远物深度线索）

| Cue | 机制 | 例子 |
| --- | --- | --- |
| Occlusion | Nearby objects occlude distant objects | 近物遮挡远物 |
| Perspective | Distant objects appear smaller | 远小近大 |
| Haze | Distant objects tend to be grayer | 远物更灰 |
| Surface texture | Texture detail decreases with distance | 纹理梯度 |

⚠️ 高频混淆点： - Convergence ≠ Accommodation：Convergence是眼球转动（肌肉），Accommodation是晶状体调焦（形状变化） - Disparity ≠ Parallax：Disparity是静态双眼图像差异，Parallax是动态观察者移动时的相对运动

考试标准句： > Our brain uses the disparity of an object observed by the two eyes to estimate the depth of the object. Parallax refers to the different amounts of movement between nearby and distant objects, as observed by a moving viewer.

1.4 Visual Acuity & Retina Display / 视觉敏锐度与视网膜显示

- Human resolution limit：~1 arc minute（1/60度）

- Retina display requirement per eye：(60 × 150°) × (60 × 135°) = 9000 × 8100 pixels

- 考试结论：True retina-level VR displays are expensive because wide FOV + high resolution imposes huge rendering and memory cost. Manufacturing such large FOV displays is also not easy.

2. HMD Image Formation（头戴显示器成像）

2.1 成像原理

| 组件 | 功能 |
| --- | --- |
| Small LCD screen | 物理屏幕很小，距眼仅几英寸 |
| Special optics/lens | 放大图像为更大、更远的virtual image |
| 作用1 | Let eyes focus at short distances without fatigue |
| 作用2 | Magnify small displayed image to fill user’s FOV |

放大公式：

Magnification M = h/h' = f/(d' - f)
Virtual image distance: d = f·d'/(d' - f)

2.2 HMD Comparison（了解层级）

| Device | Resolution/eye | Horizontal FOV | Price | Tracking |
| --- | --- | --- | --- | --- |
| PlayStation VR2 | 2000×2040 | ~110° | $600 | 6DOF, gyro+accel, 4 cameras |
| Hololens 2 | 2048×1080 | ~106° | $3,500 | 6DOF SLAM, 3 cameras per controller |
| Meta Quest Pro | 1800×1920 | 96° | $1,500 | Eye-tracking, hand tracking |
| Oculus Quest 2 | 1832×1920 | 96° | $400 | 6DOF inside-out, 4 cameras |

AR Glasses：Microsoft Hololens, Google Glass, Apple Vision Pro - Hololens 2: Laser-based RGB projectors, Intel Cherry Trail SoC + HPU, Time-of-Flight depth sensing, IMU (accel+gyro+magnetometer)

3. Convergence-Accommodation Conflict（会聚-调节冲突）

3.1 机制（最高频考点）

Real world： > When we look at an object, our eyes converge on this object, and accommodate the ciliary muscles to focus on it. Both convergence distance and accommodation distance are the SAME.

HMD： > When viewing in an HMD, if we look at the same object, our eyes converge on this object (following its virtual depth), but they accommodate the ciliary muscles to remain focusing on the virtual image plane (fixed display distance). Convergence and accommodation are DIFFERENT.

3.2 后果

- Visual discomfort（视觉不适）

- Eye strain（眼疲劳）

- Fatigue（疲劳）

- Reduced presence（沉浸感下降）

3.3 考试标准答案

Short Question标准答法： > In HMDs, vergence follows the perceived virtual object depth, but accommodation remains on the fixed display/virtual image plane. This convergence-accommodation conflict causes visual discomfort and is a fundamental limitation of current VR displays.

Long Question连接： > This conflict is one of the main reasons why prolonged HMD use causes fatigue, and why varifocal displays or light-field displays are active research topics to solve this problem.

4. Display Types Comparison（显示类型比较）

4.1 五大类型总表

| Type | Core Feature | Advantages | Disadvantages |
| --- | --- | --- | --- |
| HMD | Personal immersive, head-tracked | Most immersive, wide FOV possible | Heavy, resolution/FOV trade-off, convergence-accommodation conflict, cables |
| Hand-held Display (HHD) | Portable, periodic viewing | Flexible, can go in/out of simulation | Less immersive, small screen |
| Autostereoscopic | Glasses-free stereo | No glasses needed | Horizontal resolution halved, fixed/passive viewing zone, active version has mechanical noise |
| Large-volume | Multi-user, monitor/projector-based | Natural collaboration, large workspace | Expensive, synchronization/calibration complex |
| Volumetric | True 3D volume, 360° view | All-around view without glasses | Special hardware, limited resolution/complexity |

4.2 Autostereoscopic Displays（裸眼立体显示）

原理： > Special column-interlaced image format: odd columns = left eye view, even columns = right eye view. Left eye sees only odd columns, right eye sees only even columns.

Passive Autostereoscopic： - User’s head NOT tracked - Restricts user’s eyes to small area (stereo viewing cone) - Relation between backlighting distance (d) and user distance (D) determines viewing zone

Active Autostereoscopic： - Cameras track user’s head motion - Prism mask translated by stepper motor based on head position - Disadvantages: potential for breakage, noisy operation

分辨率代价：Horizontal resolution reduced by half（左右眼各用一半列）

代表：DTI 2018XL Virtual Window - Monoscopic: 1280×1024 - Stereoscopic: 640×1024

4.3 Large-Volume Displays（大空间显示）

Monitor-based

Active Shutter Glasses系统： 1. Stereo-ready monitor refreshes at 120-140Hz（双倍正常扫描率） 2. Computer sends two alternating offset images 3. IR emitter synchronizes display signals with active glasses 4. LC shutters alternately occlude left/right eye 5. Brain fuses quick sequence into stereo image

特点： - Flicker-free due to short shutter time + 120Hz refresh - Images appear darker（shutters filter out light） - Direct line of sight needed between IR emitter and receiver

Multi-Panel Displays： - Multiple monitors side-by-side to increase FOV - Example: PV290 (Panorama Technologies) – 3 LCD panels, 3840×1024 - Latest: 49” diagonal, 5120×1440, single display card

Projector-based

CRT vs Digital Projectors：

|  | CRT Projectors | Digital Projectors |
| --- | --- | --- |
| 技术 | Three tubes (RGB) | LCD or DLP |
| 分辨率 | 1280×1024, 120Hz | Higher |
| 亮度 | Low luminance | Much higher luminance |
| 缺点 | Need enclosures | — |

LCD vs DLP：

|  | LCD Projectors | DLP Projectors |
| --- | --- | --- |
| 技术 | 3 separate LCD glass panels (RGB) | Digital Micromirror Device (DMD) |
| 像素 | Pixels allow/block light passage | Tiny mirrors tilt ±10° by electric current |
| 优点 | More light efficient, sharper images | Smoother, less pixellation, higher contrast |
| 缺点 | Pixellation visible, bulky, dead pixels possible | — |

DMD结构： - Array of small aluminum mirrors - Each supported by two posts and torsion hinges - 5V potential electrostatically deflects mirror by ±10°

Immersive Workbench： - Projector → mirror → diffuser screen (upward reflection) - Several viewers with active glasses see 3D objects floating above

CAVE： - Multiple mirrors and projectors on walls including floor

Walls and Domes： - Projector arrays for large images accommodating many viewers - Need: large image size + brightness + high resolution - Seamless blending: Overlap ~25% of adjacent images, preprocess luminosity at overlapping areas using 2D look-up table

V-Dome： - 360° FOV, 400-500 viewers - Multiple projectors radially around semispherical rear-projection screen - Special optics pre-distort image for curved screen

4.4 Polarization（偏振）

- Pair of polarized projectors

- Polarized glasses with differently polarized lenses for each eye

- Each eye sees matching polarization component

- Inexpensive glasses

4.5 Volumetric Displays（体显示）

- 360° all-encompassing view

- Example resolution: 768×768 (pixels) × 198 (slides)

- Texas Instruments DMD chip + rotating mirrors + translucent screen

- 2D projection engine: 5000 frames/sec

- Screen rotation: 730 radians/minute

5. Tutorial 4 关联：Parallax Barrier（视差屏障）

目标：Left eye sees left image, right eye sees right image

原理： - Parallax barrier with aperture blocks light selectively - Barrier positioned between pixels and viewer

Optimum viewing position： - Fixed distance and location - Determined by barrier aperture spacing and pixel pitch

偏离后果： - Inverse image: Right eye sees left image, left eye sees right image - Blended zones: One eye sees both images → blurred/double vision

考试标准句： > The parallax barrier creates glasses-free 3D by selectively blocking pixels for each eye. However, it has an optimum viewing position; deviation causes inverse images or blended zones where one eye sees both views.

6. M3 一页纸总结 / One-Page Summary

HUMAN VISUAL:
  Fovea = high-resolution center (color)
  Peripheral = low-resolution but motion-sensitive
  Rods = night vision; Cones = color (L/M/S, M most sensitive)

FOV:
  Single eye: 150°H × 135°V
  Both eyes: 180°H × 135°V
  Binocular overlap: 120°H (stereo depth zone)

DEPTH CUES:
  Near = convergence + accommodation + disparity + parallax
  Far = occlusion + perspective + haze + surface texture

RETINA DISPLAY:
  1 arc min resolution → 9000×8100 pixels/eye → expensive

HMD:
  Lens magnifies small LCD to large virtual image
  BUT convergence-accommodation conflict = fundamental problem

DISPLAY TYPES:
  HMD (most immersive) / Hand-held (portable) / Autostereoscopic (no glasses, half resolution) / Large-volume (multi-user, expensive) / Volumetric (360°, hardware complex)

第三部分：OUTPUT DEVICES - SOUND AND HAPTICS（声音与触觉反馈）

Chapter 4 - 对应课件：4 - Output Devices (Sound and Haptics).pdf

1. Sound Properties（声音物理属性）

| 属性 | 英文 | 中文 | 数值/要点 |
| --- | --- | --- | --- |
| 本质 | Pressure wave | 压力波 | Propagating through a medium |
| 速度公式 | c = √(K/ρ) | 声速 = √(刚度/密度) | K=coefficient of stiffness, ρ=density |
| 空气中声速 | Speed in air | — | ~343 m/s (767 mph) |
| 波动特性 | Wave properties | — | Reflection（反射）, Refraction（折射）, Diffraction（衍射/绕射） |

考试标准句： > Sound is a pressure wave propagating through a medium. It has ordinary wave properties such as reflection (bouncing), refraction (change of angle when entering a different density medium), and diffraction (bending around obstacles).

2. Human Auditory System（人耳听觉系统）

| 结构 | 英文 | 功能 | VR关联 |
| --- | --- | --- | --- |
| 耳廓 | Pinna | Reflects sound, amplifies/attenuates frequencies | Provides direction information, critical for elevation cues |
| 耳道 | Ear canal | Amplifies 3-12kHz | Enhances speech-critical frequencies |
| 中耳 | Middle ear | Delicate bones magnify sound pressure | Mechanical transmission |
| 内耳 | Inner ear | Converts pressure to brain signals | Final perception |
| 可听范围 | Audible range | 20Hz - 20kHz | — |
| 最敏感频段 | Most sensitive | 3-4kHz | Design 3D sound with emphasis |
| 低频/高频敏感度 | Low/high frequency | Rather insensitive | Less perceptual resolution at extremes |

考试标准句： > Sound waves are reflected by the pinna, which amplifies some frequencies and attenuates others. These changes provide added information for the brain to determine the direction of sound.

3. 3D Sound vs Stereo Sound（核心区分）

|  | Stereo Sound | 3D Sound |
| --- | --- | --- |
| 定义 | Two-channel audio | Audio that changes with user’s head location and orientation |
| 头部追踪 | No | Yes |
| 沉浸感 | Limited | High presence |
| 技术 | Fixed left/right channels | HRTF + head tracking + real-time convolution |

考试标准答案： > The difference between stereo sound and 3D sound is that in 3D sound, the sound changes according to the location and orientation of the user’s head.

意义：3D sound让用户在VR中转动头或移动时，声音源位置感觉固定在世界中，增强presence。

4. Sound Localization Cues（声音定位线索）—— 必背

4.1 三类线索总表

| Cue Type | Purpose | Mechanism | Key Terms |
| --- | --- | --- | --- |
| Azimuth cues | Horizontal angle (left/right) | ITD + IID | Interaural differences |
| Elevation cues | Vertical angle (up/down) | Pinna filtering | Outer ear reflection |
| Distance cues | Distance (near/far) | Loudness + Prior knowledge + Motion parallax | Intensity + experience + head movement |

4.2 ITD vs IID（最高频混淆点）

|  | ITD (Interaural Time Difference) | IID (Interaural Intensity Difference) |
| --- | --- | --- |
| 中文全称 | 双耳时间差 | 双耳强度差 |
| 机制 | 声音先到达近耳，两耳到达时间差Δt | 近耳听到更强声音，远耳被头部遮挡衰减 |
| 频率依赖 | 低频更重要（长波长绕过头部） | 高频更明显（短波长被头部遮挡） |
| 物理基础 | 声速固定，距离差→时间差 | Head-shadow effect（头部阴影效应） |
| 失效情况 | <250Hz时room reverberation混淆 | — |

ITD补充： > For very low frequency sound (below 250Hz), room reverberations confuse the auditory system and ITD cannot be estimated. The brain adapts by focusing at the beginning of sound transients and discarding low-frequency steady-state information.

考试标准句： > For lower frequency sound, the ITD dominates the identification of angular location. For higher frequencies, the IID due to the head-shadow effect is more detectable.

4.3 Elevation Cues：Pinna Filtering

问题： > If the head is modeled as having simple holes for the ears, there are source locations where time and intensity cues are the same. This results in perceptual reversal, or front-back confusion.

解决： > In reality, the outer ear (pinna) is very important. Sound is reflected by it and guided to the inner ear. The direct sound and the sound reflected at the pinna interfere. Some frequencies are amplified, while others are attenuated. The sound changes according to the elevation angle.

考试标准句： > The pinna reflects and filters sound, creating elevation-dependent frequency changes that help distinguish front from back and up from down.

4.4 Distance Cues

| Mechanism | Explanation | Example |
| --- | --- | --- |
| Loudness | 近大远小 | Same source, farther = quieter |
| Prior knowledge | 结合已知声源特性 | Whisper vs Siren |
| Motion parallax | 头部移动时声源方向变化 | Head rotation changes perceived direction more for nearby sources |

5. HRTF - Head-Related Transfer Function（头相关传递函数）

5.1 定义

英文： > HRTF describes how sound from a source is transformed before reaching a listener’s ear. It captures the filtering effect of the head, pinna, and torso.

中文： HRTF描述声音从声源到达听者耳朵之前的变换过程，捕捉头部、耳廓和躯干的滤波效应。

5.2 测量方法

- Microphone at sound source location（记录原始声音S）

- Miniature microphone in inner ear（记录到达耳朵的声音）

- Compare → calculate transfer function

- Fourier transform → HRTF (H)

5.3 卷积过程 / Convolution

Yi = Hi ⊗ S

- Yi = 用户i听到的声音

- Hi = 用户i的HRTF

- S = 输入声源

- ⊗ = convolution operator

过程： > Once the HRTF is experimentally determined, it is possible to take any input sound, apply the transform, and play back the output sound to the same user over a headset. The user then has the sensation of hearing that sound as coming from the original source.

5.4 个性化特征

| 条件 | Front-Back Reversal Error Rate |
| --- | --- |
| Real sound | 5% |
| Own HRTF | 10% |
| Someone else’s HRTF | 30% |

考试结论： > Each person has his/her own HRTF signature, as no two persons have exactly the same outer ear and torso geometry. Spatial recognition drops when someone else’s HRTF is used.

5.5 System Example：Convolvotron

- 1988年Crystal River Engineering为NASA开发

- 第一个virtual 3D sound generator

- Head tracker data → host computer → HRTF lookup table → convolver processing

5.6 Speaker-Based 3D Sound（vs Headphones）

|  | Headphones | Speaker-based |
| --- | --- | --- |
| 隔离性 | Can isolate left/right ear sounds | Cannot isolate; each ear hears mixture of both speakers |
| Sweet spot | Not needed | User head must be inside sweet spot zone |
| HRTF应用 | Direct application | Need to solve speaker-to-ear HRTF matrix; invert H matrix |

Speaker-based公式：

Yleft = Hl,l ⊗ Sleft + Hl,r ⊗ Sright
Yright = Hr,l ⊗ Sleft + Hr,r ⊗ Sright

→ 已知Y，求S，需invert H matrix。

6. Haptic Feedback（触觉反馈）

6.1 核心区分（最高频混淆点）

|  | Tactile Feedback（触觉反馈） | Force Feedback（力反馈） |
| --- | --- | --- |
| 感觉类型 | Touch sensation | Force and torque |
| 反馈内容 | Surface geometry, roughness, temperature, vibration | Weight, inertia, resistance, actively resists motion |
| 物理本质 | Passive perception of surface properties | Active resistance or stopping of user motion |
| 设备例子 | Vibrotactile actuators, temperature gloves | PHANToM arm, CyberGrasp, haptic joysticks |
| 技术难度 | Relatively simple | More difficult (stability, safety, low latency required) |

考试标准答案： > Tactile feedback conveys real-time information on contact surface geometry, roughness and temperature. Force feedback provides real-time information on virtual object surface properties, weight and inertia. It actively resists, or even stops, the user’s contact motion.

6.2 Human Haptic Sensing（人体触觉感知）

皮肤触觉感受器 / Skin Tactile Sensors

| 感受器 | 英文 | 类型 | 频率响应 | 功能 |
| --- | --- | --- | --- | --- |
| 梅斯纳小体 | Meissner corpuscles | Fast-adapting | 50-300Hz | 检测接触力变化、vibration |
| 默克尔盘 | Merkel disks | Slow-adapting | 0-10Hz | 检测edges、skin stretch、static force |
| 帕西尼小体 | Pacinian corpuscles | Fast-adapting | 50-300Hz | 检测接触力变化（深层） |
| 鲁菲尼小体 | Ruffini corpuscles | Slow-adapting | 0-10Hz | 检测skin stretch、static force |

Slow-Adapting vs Fast-Adapting（必背区分）

|  | Slow-Adapting | Fast-Adapting |
| --- | --- | --- |
| 代表 | Merkel disks, Ruffini corpuscles | Meissner corpuscles, Pacinian corpuscles |
| 频率 | 0-10Hz | 50-300Hz |
| 响应特性 | Maintain constant discharge for constant force | Drop discharge fast under constant force |
| 功能 | Detect quasi-steady state: edges, stretch, static force | Detect change of contact force, vibration |
| 记忆法 | “Slow = Static” | “Fast = Fluctuation” |

考试标准句： > Slow-adapting sensors (Merkel and Ruffini) maintain a constant rate of discharge for a long time in response to a constant force, used to detect edges, skin stretch, and static force. Fast-adapting sensors (Meissner and Pacinian) detect high-frequency stimuli and mainly sense the change of contact force, dropping their discharge rate so fast that constant force becomes undetected.

空间分辨率 / Sensing Resolution

| 部位 | 最小分辨距离 |
| --- | --- |
| Fingertips（指尖） | 2.5mm |
| Palm（手掌） | 11mm |

时间阈值： > Skin mechanoreceptors have a successiveness threshold of only 5msec, which is much smaller than the corresponding value for the eye (25 msec).

温度感知 / Temperature Sensing

- Thermoreceptors：表皮（冷感）/ 真皮（温感）

- Nociceptors：感知-15°C到45°C范围外 → 触发痛觉

本体感觉 / Proprioception

定义：The perception of one’s own body position and motion.

分辨率： | 关节 | 分辨率 | |——|——–| | Hip | 0.2° (highest) | | Shoulder | 0.8° | | Finger | 2.5° | | Toe | 6.1° (lowest) |

原理： > The proprioceptive resolution is highest for the hip and shoulder joints and lowest for the finger and toe. This reflects the body’s need to minimize errors at the limb extremities, which are most dependent on the sensitivity of shoulder and hip proprioceptive sensors.

6.3 Tactile Feedback Devices（触觉设备）

| 设备 | 原理 | 反馈类型 | 特点 |
| --- | --- | --- | --- |
| Tactile Mouse / iFeel Mouse | Electrical actuator vibrates mouse shell | Vibrotactile | Adds cue without looking at display |
| CyberTouch Gloves | CyberGlove + 6 vibrotactile actuators (DC motor + off-centered mass) | Vibrotactile per finger | Retrofitted to CyberGlove |
| Temperature Feedback Gloves | Thermoelectric heat pump (Peltier effect: N/P semiconductor + ceramic insulator) | Temperature | DC current → temperature difference |
| Temperature Sensing System (C&M) | Heat pump + thermocouple sensor; compare finger temp with target; adjust current | Closed-loop temperature | Feedback-controlled |

温度反馈原理： > When a DC current is applied, P and N charges move towards the heat sink plate. This causes a drop in temperature of the heat source plate and a rise in temperature of the heat sink plate. The larger the current, the higher the temperature difference.

6.4 Force Feedback Devices（力反馈设备）

| 设备 | DOF | 特点 | 力输出 | 缺点 |
| --- | --- | --- | --- | --- |
| Force Feedback Joysticks (WingMan Force 3D) | 2 rotational | 最便宜、最广泛 | Rotational torque at origin | — |
| PHANToM Arm (Desktop) | 6 (3 active translational + 3 passive orientation) | 高精度、hand feeling | Peak 6.4N, Continuous 1.7N | — |
| PHANToM 1.5/6.0 | 6 (with active torques) | Larger forces | Max 8.5N, Torque 170-515 mN-m | — |
| Haptic Master Arm | 3 cylindrical | 高力输出、高刚度 | Max 250N, Stiffness 5×10⁴ N/m | Larger inertia, expensive |
| CyberGrasp | Per-finger force | Individual finger control | Max 16N/finger, weight 539g | Fatigue |
| CyberForce | Wrist force | Mechanical arm + CyberGrasp | Max 8.8N at wrist | No torques |

Trade-off规律： > Higher force / stiffness → larger inertia → heavier/more expensive → more user fatigue.

7. M4 一页纸总结 / One-Page Summary

SOUND:
  Pressure wave, 343m/s in air, reflection/refraction/diffraction
  Human: pinna helps direction, ear canal amplifies 3-12kHz, most sensitive 3-4kHz

3D SOUND ≠ STEREO:
  3D changes with head location/orientation; stereo is fixed

LOCALIZATION:
  Azimuth (水平) = ITD (低频时间差) + IID (高频强度差/head-shadow)
  Elevation (垂直) = pinna filtering (频率整形)
  Distance = loudness + prior knowledge + motion parallax

HRTF:
  Describes sound transformation from source to ear
  Personalized; wrong HRTF → front-back confusion increases (5%→10%→30%)
  Yi = Hi ⊗ S (convolution)

HAPTIC:
  Tactile = touch (geometry/roughness/temperature/vibration)
  Force = weight/inertia/resistance, actively resists motion

SKIN SENSORS:
  Slow-adapting (Merkel/Ruffini, 0-10Hz, static force/edges)
  Fast-adapting (Meissner/Pacinian, 50-300Hz, vibration/change)
  Fingertip resolution: 2.5mm; Palm: 11mm; Time threshold: 5ms (vs eye 25ms)

PROPRIOCEPTION:
  Hip/shoulder: highest resolution (0.2°/0.8°)
  Finger/toe: lowest (2.5°/6.1°)

第四部分：跨章节连接与长题素材

1. 核心连接矩阵

| 连接概念 | Ch2 Input | Ch3 Visual Output | Ch4 Audio/Haptic Output | Architecture | Presence |
| --- | --- | --- | --- | --- | --- |
| Latency | Tracker latency | Graphics rendering latency | Haptic rendering needs ~1kHz | Pipeline bottleneck | Causes sickness |
| Tracking | 6DOF position/orientation | Determines Mview | Head tracking for 3D sound | Application stage update | Enables interaction |
| FOV | — | Large FOV increases presence | — | Rendering cost increases | Visual immersion |
| Depth | — | Convergence-accommodation conflict | — | — | Visual comfort |
| Multi-sensory | Multiple input channels | Visual + stereo | 3D audio + haptic feedback | Decoupled pipelines | Maximizes presence |
| Personalization | IMU calibration | IPD adjustment | HRTF personalization | — | Individual comfort |
| Resolution/Rate | Update rate >30Hz | Retina: 9000×8100/eye | Haptic: ~1kHz | Synchronization | Realism |

2. Long Question 万能模板

模板：Design a VR System for X Application

1. APPLICATION GOAL
   - What problem does the VR system solve?
   - Why VR is useful compared with real-world training/practice?
   - Target users?

2. INPUT DEVICES
   - What needs tracking? (head/hand/tool/body)
   - Tracker type choice + justification (accuracy, latency, range, occlusion, intrusiveness)
   - Navigation/manipulation devices?

3. OUTPUT DEVICES
   - Visual: HMD / large-volume / autostereoscopic / monitor
   - Audio: 3D sound if direction/distance matters (HRTF + head tracking)
   - Haptics: tactile or force feedback if touch/force matters

4. MODELING (Ch5/6)
   - Geometric: meshes/splines/point cloud
   - Physical: collision detection, deformation, force model
   - Behavior: state machine / agents / crowd
   - Management: LoD, texture mapping, portals

5. ARCHITECTURE (Ch5)
   - Graphics pipeline: application → geometry → rasterizer
   - Haptics pipeline if needed: collision → force computation → output
   - Bottleneck identification and optimization
   - Graphics vs haptics synchronization (decouple but sync at application stage)

6. PRESENCE (Ch7)
   - Increase: high visual quality, large FOV, low latency, tracking, multimodal feedback
   - Decrease: sensory mismatch, high latency, poor interaction, cables

7. DISTRIBUTED ISSUES (Ch9, if multi-user)
   - DVE/CVE architecture
   - Partitioning strategy (user/region/dynamic)
   - Latency handling: rollback, prediction, dead reckoning

8. BENEFITS & LIMITATIONS
   - Benefits: safety, repeatability, lower cost, rare cases, remote access
   - Limitations: latency, accuracy, cost, sickness, training transfer, calibration

示例：VR Medical Training System

Input： > Track surgical tools and user hands using optical tracker for precision (but beware occlusion); mechanical/haptic device (e.g., PHANToM) for tool force feedback.

Output： > Stereo visual display/HMD with large FOV; force feedback through PHANToM-like device for tissue resistance; optional 3D audio for equipment sounds.

Architecture： > Graphics pipeline for rendering at 60fps; haptics pipeline for collision detection and force computation at ~1kHz; decouple but synchronize at application stage.

Presence： > Low latency, accurate tracking, realistic haptics, high visual quality, ability to see user’s own hands (body visibility).

Trade-off： > High computation cost, haptic realism difficult to achieve, calibration requirements, equipment cost, potential convergence-accommodation fatigue.

3. 高频混淆点速查表 / Confusion Quick Reference

| 混淆对 | 区分要点 | 所属章节 |
| --- | --- | --- |
| Jitter vs Drift vs Latency | Jitter=静止随机抖; Drift=时间累积偏; Latency=动作到反馈延迟 | Ch2 |
| Accuracy vs Precision | Accuracy=接近真实值; Precision=重复一致性 | Ch2 |
| 2DOF vs 3DOF vs 6DOF | 2=平面; 3=位置或方向; 6=位置+方向 | Ch2 |
| AC vs DC Magnetic | AC=交变场, eddy current严重; DC=脉冲直流, 减轻但怕铁磁金属 | Ch2 |
| Outside-in vs Inside-out | 摄像头在外/在内; marker在人上/在环境上 | Ch2 |
| Gyro vs Accelerometer | Gyro=角速度,短期准长期漂; Accel=加速度,长期稳短期噪 | Ch2 |
| Convergence vs Accommodation | Convergence=双眼会聚(肌肉); Accommodation=晶状体调焦(形状) | Ch3 |
| Disparity vs Parallax | Disparity=静态双眼图像差; Parallax=动态观察者移动相对运动 | Ch3 |
| ITD vs IID | ITD=时间差(低频重要); IID=强度差(高频重要,head-shadow) | Ch4 |
| Stereo vs 3D Sound | Stereo=固定; 3D=随头部pose变化 | Ch4 |
| HRTF Personalized vs Generic | Own=10% error; Other’s=30% front-back reversal | Ch4 |
| Tactile vs Force Feedback | Tactile=被动感知表面; Force=主动抵抗运动 | Ch4 |
| Slow vs Fast-Adapting | Slow=持续响应constant force; Fast=只响应变化 | Ch4 |
| Meissner vs Merkel | Meissner=fast, vibration; Merkel=slow, edges/static | Ch4 |
| Local vs Global Illumination | Local=单个物体; Global=考虑物体间反射 | Ch5/6 |
| Flat vs Gouraud vs Phong | Flat=每面一色; Gouraud=顶点插值; Phong=像素插值法线 | Ch5/6 |
| User vs Region vs Dynamic Partitioning | User=按用户分; Region=按空间固定分; Dynamic=运行时调整 | Ch9 |
| Rollback vs Prediction | Rollback=状态回滚修正; Prediction=预测未来位置 | Ch9 |

4. 必背标准答案句 / Standard Answer Sentences

Short Question 模板句

Define virtual reality： > Virtual Reality is a human-computer interface that supports real-time simulation and interaction through multiple sensory channels, such as visual, auditory and tactile channels.

Explain four levels of VR： > Passive VR allows minimal user control (like watching a movie); Exploratory VR allows navigation but not modification; Interactive VR allows both exploration and object manipulation; Collaborative VR supports multiple users performing tasks together in the same virtual world.

Explain tracking performance metrics： > Accuracy measures how close the tracked value is to the actual position. Jitter refers to random output fluctuations when the object is stationary. Drift is the growing error over time due to accumulation. Latency is the delay between motion and feedback, which can cause simulation sickness.

Compare magnetic and optical trackers： > Magnetic trackers have no line-of-sight restriction and are lightweight, but are affected by metallic objects and have limited range. Optical trackers offer high precision and are standard for motion capture, but suffer from occlusion and require calibration.

Explain convergence-accommodation conflict： > In HMDs, the eyes converge to the perceived virtual object depth, but accommodate to the fixed display plane distance. This mismatch causes visual discomfort and is a fundamental limitation of current VR displays.

Explain ITD and IID： > ITD (Interaural Time Difference) is the time delay between sound reaching the two ears, more important for low frequencies. IID (Interaural Intensity Difference) is the loudness difference due to the head-shadow effect, more detectable for high frequencies.

Explain HRTF： > HRTF (Head-Related Transfer Function) describes how sound from a source is transformed by the listener’s head, pinna, and torso before reaching the ear. It is personalized; using someone else’s HRTF increases front-back reversal errors from 10% to 30%.

Compare tactile and force feedback： > Tactile feedback conveys surface geometry, roughness, and temperature. Force feedback provides weight, inertia, and resistance, actively resisting the user’s motion. Force feedback is technically more challenging due to stability and safety requirements.

Explain graphics pipeline stages： > The graphics rendering pipeline has three stages: Application stage (CPU: read input, update world, select geometry); Geometry stage (GPU: transformations, shading, projection, clipping); Rasterizer stage (GPU: fragments, anti-aliasing, texture mapping).

Explain presence factors： > Presence is increased by high visual quality, large FOV, low latency, accurate head tracking, multimodal feedback (audio, haptics), and interactivity. It is decreased by sensory mismatch, high latency, poor interaction, cables, and inability to see one’s own body parts.

5. Two-Level MCQ 解题策略

步骤： 1. 遮住第二层，先判断第一层Statement是否正确 2. 对每个选项问： - 定义是否错误？ - 是否混淆了两个概念？ - 方向是否反了？ - 是否绝对化了？ 3. 第一层选定后，再看第二层Reason是否正确 4. 判断Reason是否真正解释第一层（不是相关就算）

常见陷阱模式： - “Because A causes B” → 但其实是C causes B - “X is better than Y” → 但题目问的是特定场景，Y更好 - “All trackers need line-of-sight” → Mechanical和Magnetic不需要 - “HRTF is the same for everyone” → 恰恰相反，高度个性化

附录：考前2小时冲刺清单

必须能默写的10个概念

- VR definition (real-time + interaction + multiple sensory)

- Four levels of VR (passive → exploratory → interactive → collaborative)

- Tracking 4 metrics (accuracy, jitter, drift, latency)

- Six tracker types + each pros/cons

- IMU 3 sensors complementary relationship

- FOV numbers (150°/135° single, 180°/120° overlap)

- Depth cues (near: convergence/accommodation/disparity/parallax; far: occlusion/perspective/haze/texture)

- Convergence-accommodation conflict mechanism

- 3D sound cues (ITD/IID/pinna/loudness/motion parallax) + HRTF personalization

- Tactile vs Force feedback distinction

必须能画出的3个图

- Coordinate transformation chain (Mworld → Mview → Mproj)

- Graphics pipeline 3 stages (Application → Geometry → Rasterizer)

- VR system loop (Input → Update World → Render → Output)

必须能比较的5组概念

- Mechanical vs Magnetic vs Optical trackers

- AC vs DC magnetic trackers

- Outside-in vs Inside-out optical

- Convergence vs Accommodation

- Tactile vs Force feedback

最后提醒：VR不是一堆设备名，而是一套实时、多感官、人机交互系统。考试围绕这套系统来问。每个答案都要回到：User action → Input tracking → World update → Rendering → Output feedback → Presence → Goal achievement。

文稿编制完成 | 基于复习路径.md + 复习Prompt.md + Ch2/3/4 PDF课件 适用考试：CS4188/CS5188 Virtual Reality Technologies and Applications 期末考试